2DNMR-Analyst

User Guide v2.0

# 🔍 Introduction

2DNMR-Analyst is a Shiny application for loading, visualizing, processing and exporting 2D NMR spectra from Bruker data. It provides a complete interactive interface for metabolomics analysis.

## Main Features

- Interactive visualization of 2D spectra (TOCSY, COSY, HSQC, UFCOSY)

- Automatic peak detection with or without clustering (DBSCAN)

- Manual creation and modification of peaks and bounding boxes

- Fusion of multiple peaks into one

- Integration with Sum, Gaussian, or Voigt fitting methods

- Fit Quality analysis with R² metrics and residuals visualization

- Import/Export of data (peaks, boxes, sessions)

- Batch processing across multiple spectra

# 💻 Installation & Access

## Download the Source Code

The source code is available on GitHub:

https://github.com/JulienGuibertTlse3/2DNMR-Analyst

### Option A: Direct Download (ZIP)

- Go to the GitHub link above

- Click the green "Code" button

- Select "Download ZIP"

- Extract the archive to your preferred folder

### Option B: Clone with Git

git clone https://github.com/JulienGuibertTlse3/2DNMR-Analyst.git

## Prerequisites

- R (version ≥ 4.0 recommended) - https://cran.r-project.org/

- RStudio (recommended) - https://posit.co/download/rstudio-desktop/

## File Structure

2DNMR-Analyst/ ├── run_app.R          # Entry point - run this file ├── Shine.R            # Main application └── Function/     ├── Read_2DNMR_spectrum.R   # Bruker data reading     ├── Vizualisation.R         # Visualization and clustering     ├── Peak_picking.R          # Peak detection     └── Peak_fitting.R          # Gaussian/Voigt fitting

## Launching the Application

✅ Simple method (recommended): Use the run_app.R script

- Open RStudio

- Open the run_app.R file

- Click "Source" or press Ctrl+Shift+Enter

The script will automatically install all required packages and launch the application.

# 🧭 Application Navigation

The application is organized into two main tabs:

| Tab | Description |
| --- | --- |
| Guide | Introduction, tool presentation and contextual help |
| Analysis | Main interface: interactive visualization and spectrum analysis |

# 📂 1. Load Data

## Bruker File Structure

Bruker files must follow this structure:

/<spectrum_folder>/pdata/1/ containing ser or fid

## Loading Steps

- Go to the Analysis tab

- Click on the "1. Load Data" section

- Click "Select Folder"

- Select the folder containing your Bruker 2D spectra

- Check the spectra you want to load

- Click "Load Selected"

💡 Tip: Use "Reset All" to completely reset the application if needed.

# 📈 2. Plot Settings

## Spectrum Type

- TOCSY: Homonuclear correlations through bonds

- COSY: Direct homonuclear correlations

- HSQC: Heteronuclear ¹H-¹³C correlations

- UFCOSY: Ultrafast COSY

## Parameters

- Threshold: Intensity threshold for contour display

- Auto: Automatically calculates an optimal threshold

- Advanced: Advanced options (noise threshold type, value, etc.)

Click "Generate Plot" to display the spectrum.

# 🎯 3. Peak Picking

## Available Methods

Local Max: Detection by local maxima with DBSCAN clustering

CNN: Detection by convolutional neural network (if available)

## Options

- No clustering: Disables DBSCAN grouping

- Epsilon (eps): Maximum distance between points of the same cluster

- Delete ranges: Eliminates the least intense peaks from a selected zone

# ✏️ 4. Manual Editing

This section contains 4 collapsible sub-sections:

## 🖱️ Click Mode

- Off: Clicks do nothing

- Add box (2 clicks): Click two corners to create a box

- Delete box on click: Click inside a box to mark it for deletion

## 🔗 Fusing Peaks and Boxes

Select multiple peaks or boxes in the Data tab (Ctrl+Click), then click "Fuse Selected" to merge them into a single element.

## 📦 Edit Selected Box

- Select a box in the graph or Data tab

- Modify coordinates (xmin, xmax, ymin, ymax)

- Use arrow buttons (↑↓←→) to move the box

- Use +/- buttons to expand/shrink

- Adjust the Step value for movement increment

- Click "Apply Edit" to confirm

## ➕ Add Manually

- Peak: Enter F2 (ppm) and F1 (ppm) coordinates

- Box: Enter the 4 coordinates (xmin, xmax, ymin, ymax)

⚠️ Important: Click "Apply" to validate changes or "Discard" to cancel them.

# 📐 5. Integration

This new section allows you to calculate peak intensities with different methods:

## Area Under Curve (AUC)

Sum: Direct integration of intensity values in the box. Fast and simple.

## Peak Fitting

Gaussian: Fits a 2D Gaussian function to the peak shape

Voigt: Fits a Voigt profile (Gaussian + Lorentzian convolution)

## Fitting Options

- Include R² in export: Adds fit quality metrics to the output

- Min R² threshold: Peaks below this threshold will use sum fallback

Click "Run Integration" to process all boxes.

# 📊 Fit Quality Tab

When using Gaussian or Voigt integration methods, a dedicated "Fit Quality" tab becomes available:

## Available Information

- Fit Summary by Method: Statistics grouped by fitting method

- Fitted Boxes Details: R² values for each box, sortable table

- R² Distribution: Histogram of fit quality across all boxes

- Selected Box - 2D Fit: Visual representation of the fitted region

- Residuals Distribution: Histogram of residuals for selected box

## Interpretation Tips

| R² Value | Interpretation |
| --- | --- |
| R² > 0.9 | Excellent fit - peak is well-defined |
| R² 0.7-0.9 | Good fit - acceptable quantification |
| R² < 0.7 | Poor fit - consider manual inspection or sum method |

# 💾 6. Save & Export

## 💼 Session

- Save: Saves the complete session (peaks, boxes, parameters) as .rds file

- Load: Restores a previously saved session

## 📥 Import Peaks & Boxes

- Peaks CSV: Import a CSV file with peak coordinates

- Boxes CSV: Import a CSV file with bounding box coordinates

## 📤 Export Data

- Peaks: Exports centroids to CSV format

- Boxes: Exports bounding boxes to CSV format

- Batch Export: Projects boxes onto all spectra and exports intensities

## CSV File Format

Peaks file: stain_id; F2_ppm; F1_ppm; stain_intensity

Boxes file: stain_id; xmin; xmax; ymin; ymax; stain_intensity

💡 Note: For batch treatment, limit the number of spectra per batch to 25 for TOCSY, 50 for COSY and HSQC.

# 📊 Data Tab

The Data tab displays detected peaks and boxes in editable tables:

- Peaks: List of centroids with F2/F1 coordinates and intensity

- Boxes: List of bounding boxes with xmin/xmax/ymin/ymax coordinates

- Click a row to select an element for editing or deletion

- Use Ctrl+Click for multiple selection

# ⏳ Pending Tab

The Pending tab shows modifications not yet applied:

- Pending Peaks: Peaks to add or delete

- Pending Boxes: Boxes to add or delete

- Pending Fusions: Peak fusions awaiting confirmation

# 💡 Tips & Best Practices

- Always start with a QC sample or the most intense spectrum to optimize parameters before batch analysis

- Use "Auto" to automatically calculate the optimal threshold

- Increase epsilon to group more peaks together

- Decrease epsilon to get more individual peaks

- Check modifications in the "Pending" tab before clicking Apply

- Use "Reset All" to start over

- For Gaussian/Voigt fitting, check the Fit Quality tab to validate results

# 🔧 Troubleshooting

| Problem | Solution |
| --- | --- |
| No spectrum detected | Check Bruker folder structure (pdata/1/) |
| Empty or error plot | Adjust threshold or use Auto |
| Too many/few peaks detected | Adjust epsilon and/or intensity threshold |
| Import fails | Check CSV format (separator ; or ,) |
| Application slow | Reduce the number of contours in Advanced |
| Poor fit quality (R² < 0.7) | Consider using Sum method or adjusting box boundaries |

────────────────────────────────────────

2DNMR-Analyst v2.0

Developed for metabolomics research

Author: Julien Guibert (julien.guibert@inrae.fr)

GitHub: JulienGuibertTlse3/2DNMR-Analyst

Last updated: February 2026